DOING PHYSICS WITH PYTHON

COMPUTATIONAL OPTICS

RAY (GEOMETRIC) OPTICS

MATRIX METHODS IN PARAXIAL OPTICS

Ian Cooper

Please email me any corrections, comments, suggestions or additions:   matlabvisualphysics@gmail.com

DOWNLOAD DIRECTORIES FOR PYTHON CODE

Google drive

GitHub

S003C.py

Functions to commute the most commonly used transformation matrices.

S003B.py

Transformation matrices: Propagation into a long cylinder with spherical end.

Translation 0  1; refraction 1 2; translation 2  3

MATRIX METHODS FOR RAY PROPAGATION

RAY-TRANSFER MATRICES

We can define matrixes for translation, reflection at plane surfaces, reflection at spherical surfaces, refraction at plane surfaces, and refraction at spherical surfaces. By combining appropriate individual matrices in the proper order, it is possible to express any optical system by a 2x2 matrix, which we call the system matrix. Table 1 gives a quick summary of some simple ray-transfer matrices.

Table 1  Simple ray-transfer matrices

The Python Code S003C.py defines the ABCD matrices for the examples given in Table 1.  These functions can be used to calculate the transform of an input column vector (y coordinate and elevation angle [slope angle] ) to the output column vector.  Inputs are often indicated by <<< or >>> in the Python Code. The results of running the Code are displayed in the Console Window as shown below.

1   Translation

y0 = 5.000   a0 =  18.000  deg

y1 = 5.628   a1 =  18.000  deg

2   Refraction at a plane interface

y0 = 5.000   a0 =  18.000  deg

y1 = 5.000   a1 =  12.000  deg

3   Reflection from a spherical mirror

y0 = 5.000   a0 =  18.000  deg

y1 = 5.000   a1 =  75.296  deg

4  Refraction at a spherical interface

y0 = 5.000   a0 =  18.000  deg

y1 = 5.000   a1 =  2.451  deg

5  Thin lens matrix

y0 = 5.000   a0 =  18.000  deg

y1 = 5.000   a1 =  8.419  deg

SIGNIFICANCE OF SYSTEM MATRIX ELEMENTS

We can now examine the significance of a zero matrix element. The transformation from input (0) to output (1) can be expressed as

A = 0

The output height (altitude)  is independent of the height of the input . Therefore, all rays departing the input plane at the same angle, regardless of height, arrive at the same height  at the output plane. The output plane thus functions as the second focal plane.

B = 0

The height is independent of . Thus, all rays from a point at height  in the input plane arrive at the same point of height  in the image plane. The points are then related as object point and image point. The input and output planes correspond to conjugate planes for the optical system. Since  the matrix element represents the linear magnification.

C = 0

The slope angle   is independent of . A set of parallel input rays produces a set of parallel output rays. The angular magnification is given by .

D = 0

The output slope angle (elevation)  is independent of the input slope angle . Hence, for a fixed value for , all rays leaving a point in the input plane will have the same angle at the output plane. The input plane thus coincides with the first focal plane of the optical system.

EXAMPLE 1        B = 0

A small object is placed at a distance of 16 mm from the left end of a long plastic rod with a spherical end of radius 4 mm. The refractive index of the rod is 1.50 and the object is in air. The output plane is at a distance x from the spherical cap.

We want to determine the image distance x and its lateral magnification. The system matrix consists of the product of three matrices: M1 (translation in air 16 mm), M2 (refraction at spherical surface, M3 (translation in rod x mm).

The Python Code S003B.py is used to answer the exercise question. The goal is to find the value of x (L23) such that the matrix element B is zero. When B = 0, the rays from an object point pass through the one image point as shown in figure 1.

The answer is x = 24, and the linear magnification is -1 (A = -1).

A summary of the simulation parameters is displayed in a the Console Window and ray paths are displayed in a Figure Window.

Fig. 1. B = 0 when x = 24. All rays from a point at height  in the input plane arrive at the same point of height  in the image plane.

The Python Code S003B.py calculates each transformation so the path of the ray can be plotted and the image point is also calculated using the system matrix.

Example 1: propagation into long cylinder

Object plane  L01 = 16.00

Image plane  x = L23 = 24.00

Input vector:  y0 = 1.000   a0 =  5.000  deg

0 --> 1   Translation

y1 = 2.396   a1 =  5.000  deg

[[ 1. 16.]

[ 0.  1.]]

1 --> 2   Refraction

y2 = 2.396   a2 =  -8.108  deg

[[ 1.          0.        ]

[-0.08333333  0.66666667]]

2--> 3   Translation

y3 = -1.000   a3 =  -8.108  deg

[[ 1. 24.]

[ 0.  1.]]

System matrix

Input vector:  y0 = 1.000   a0 =  5.000  deg

Output vector:  y1 = -1.000    a1 =  -8.108  deg

A = -1.00   B = 0.00   C = -0.08   D = -0.67

When x = 20 (L23 = 20), B = 2.67, the rays from one point on the object do not coincide in the image plane (figure 2).

Fig. 2. The three rays from one point in the object plane do not intersect at one point in the image plane when .