DOING PHYSICS WITH PYTHON

# NUMERICAL INTEGRATION

# [1D] AND [2D] INTEGRALS

Ian Cooper

matlabvisualphysics@gmail.com

DOWNLOAD DIRECTORIES FOR PYTHON CODE

Google drive

GitHub

op001.py   op002.py   op003.py

Computation of the integral of functions of the form f(x) and  f(x,y) using Simpson’s 1/3 rule. The code can be changed into a function.

COMPUTATION OF ONE-DIMENSIONAL INTEGRALS

We want to compute a number expressing the definite integral of the function f(x) between two specific limits a and b

(1)

The evaluation of such integrals is often called quadrature.

# In Python we can consider two approaches to evaluate a definite integral (equation 1). The library from scipy import integrate is required.  Integration of a function using the functions

# quad

# simps for sampled data using Simpson’s rule.

# Steps:

# Define the function and its limits

# Use the function quad or simps

# To illustrate how to compute the integral a number of examples will be given. Also, it is often a good idea to plot the graph of the f(x).

# Example 1    op003.py

# …

# Using quad function returns integral value and error estimate

# Iquad = 0.9999999999999999 1.1102230246251564e-14

# Using simps function

# Isimps = 1.000000000004289    number of sample N = 299

# Plot:  cos(x) for x = 0 to x = .

# Using Simpson’s rule: A major problem that arises with non-adaptive methods is that the number N of partitions of the function required to provide a given accuracy is initially unknown. One approach to this problem is to successively double the number of partitions, and compare the results as the number of partitions increase.

# Python code   op003.py

# import numpy as np

# from numpy import pi, sin, cos, exp, linspace

# from numpy.linalg import eig

# from scipy.integrate import odeint, quad, dblquad, simps

# from scipy import pi, sqrt

# import matplotlib.pyplot as plt

# import time

# from mpl_toolkits.mplot3d import axes3d

# import sympy as sym

# def func(x):

# f = cos(x)

# return f

# # >>>>> Inputs: Grid points, limits

# N = 299

# a = 0; b = pi/2

# # function f(x)

# x =linspace(a,b,N)

# f = func(x)

# # Integrate function uisng quad

# Iquad, Ierr = quad(func,a,b)

# print(Iquad,Ierr)

# # Integrate function using simps

# Is = simps(f,x)

# print(Is)

# # Plot function

# plt.rcParams['font.size'] = 12

# plt.rcParams["figure.figsize"] = (6,3)

# # Figure 1    t vs x

# fig1, axes = plt.subplots(nrows=1, ncols=1)

# axes.set_xlabel('x',color= 'black',fontsize = 12)

# axes.set_ylabel('f',color = 'black',fontsize = 12)

# axes.xaxis.grid()

# axes.yaxis.grid()

# xP = x; yP = f

# axes.plot(xP, yP,'b',lw = 2)

# fig1.tight_layout()

# # fig1.savefig('a1.png')

# Example 2       op003.py

# Iexact = 7.38905609893065 + 7.3890560989306495j

# Using quad function returns integral value and error estimate

# Iquad = 7.38905609893065 8.203500211279747e-14

# quad function only returns real part

# Using simps function (number of sample N = 299)

# Isimps = 7.389056098962341+7.389056098962339j

# SYMBOLIC [1D] INTEGRATION

# Integrals can be evaluated symbolically. You need the library import sympy as sym to use any of the symbolic tools.

# A good reference is

# https://scipy-lectures.org/packages/sympy.html#integration

# Code from op003.py

# #%%  Symoblic integration: select function by removing #

# x = sym.Symbol('x')

# y = sym.Symbol('y')

# # y = 6 * x ** 5

# # y = sym.sin(x)

# # y = sym.log(x)

# # y = 2 * x + sym.sinh(x)

# sym.integrate(y, x)

# # Highlight the code and use F9 to execute the code

# # It is possible to compute definite integral:

# # y = x**4;  a = -1; b = 1; sym.integrate(y, (x, a, b))

# # sym.integrate(sym.sin(x), (x, 0, sym.pi))

# # sym.integrate(sym.cos(x), (x, -sym.pi / 2, sym.pi / 2))

# # Also improper integrals are supported as well:

# # sym.integrate(sym.exp(-x), (x, 0, sym.oo))

# # sym.integrate(sym.exp(-x ** 2), (x, -sym.oo, sym.oo))

# [2D], surface, double, integration

We want to compute the value of a definite integral of the function f(x,y) between two specific limits (ax, bx) and (ay, by)

A very practical and versatile way to compute [2D] integrals is using a [2D] form of Simpson’s 1/3 rule.

# Simpson’s 1/3 rule

This rule is based on using a quadratic polynomial approximation to the function f(x) over a pair of partitions.  N-1 is the number of partitions where N must be odd and

x  h = (b – a) / (N-1). The integral is expressed below and is known as the composite Simpson’s 1/3 rule.

Simpson’s rule can be written vector form as

where .

c and f are row vectors and fT is a column vector.

# Simpson’s [2D] method

The double integral

can be approximated by applying Simpson’s 1/3 rule twice – once for the x integration and once for the y integration with N partitions for both the x and y values.

x-values:

y-values:

The lower and upper bounds determine the size of the partitions

The N x-values and N y-values form a two-dimensional grid of N x N points. The function f(x,y) and the two-dimensional Simpson’s coefficients are calculated at each grid point. Hence, the function f(x,y) and the two-dimensional Simpson’s coefficients can be represented by N x N matrices F and S respectively.

The Simpson matrix S for N = 5 is

| 1 x 1 = 1 | 4 x 1 = 4 | 2x1 = 2 | 4x1 = 4 | 1x1 = 1 |
| --- | --- | --- | --- | --- |
| 1 x 4 = 4 | 4 x 4 = 16 | 2x4 = 4 | 4x4 = 16 | 1x4 = 1 |
| 1 x 2 = 8 | 4 x 2 = 8 | 2x2 = 4 | 4x2= 8 | 1x2 = 1 |
| 1 x 4 = 4 | 4 x 4 = 16 | 2x4 = 4 | 4x4 = 16 | 1x4 = 1 |
| 1 x 1 = 1 | 4 x 1 = 4 | 2x1 = 2 | 4x1 = 4 | 1x1 = 1 |

1      4     2      4     2     4      2     4       1

4    16     8    16     8    16     8    16     4

2      8     4      8     4      8     4     8       2

4    16     8    16     8    16     8    16     4

2      8     4      8     4      8     4      8      2

4    16     8     16     8   16     8    16     4

2      8     4       8     4     8     4      8      2

4    16     8     16     8   16     8    16     4

1      4     2       4     2     4     2      4      1

Therefore, the two-dimensional Simpson’s rule which is used to estimate the value of the surface integral can be expressed as

We include a mask matrix M (NxN) which has elements equal to 1 or 0 only. The mask matrix M is used where the integration is  not over a rectangular area defined by the limits ax, bx, ay and by. The integral then becomes

Required steps using the Simpson [2D] method are:

- Define the matrix S for the [2D] Simpson coefficients.

- Define the x and y ranges.

- Define the xy mesh for xx and yy using the meshgrid function.

- Calculate the matrix function F.

- Calculate the mask matrix M.

- Redefine the matrix F

F = M F

- Sum all the elements of the matrix F S and multiply by        (hx hy / 9) to give the value of the integral.

We will consider a number of examples which demonstrates how to apply the two-dimensional Simpson’s rule using the code  op001.py

Example 3     op001.py

Integrate

x: 0  2   and  y: 1  5

The exact value of the integral can be found analytically and its value is 416. So, we can compare the numerical estimate with the known exact value.

This is a two-dimensional problem, so we need to specify the values (x,y) at all grid points which are determined from the upper and lower bounds. We can do this using the Python function meshgrid to calculate the value of the function f(x,y) at each grid point (x,y).

ax = 0; bx = 2; ay = 1; by = 5

x = linspace(ax,bx,N)

y = linspace(ay,by,N,N)

xx, yy = np.meshgrid(x,y)

f = xx**2*yy**3

[2D] space

To show how the meshgrid functions works, see figure (1) and the outputs of the variables x, y, xx, yy and f that can be displayed in the Console Window.

x =   0    0.5000    1.0000    1.5000    2.0000

xx =

0    0.5000    1.0000    1.5000    2.0000

0    0.5000    1.0000    1.5000    2.0000

0    0.5000    1.0000    1.5000    2.0000

0    0.5000    1.0000    1.5000    2.0000

0    0.5000    1.0000    1.5000    2.0000

y =   1     2     3     4     5

yy =

1     1     1     1     1

2     2     2     2     2

3     3     3     3     3

4     4     4     4     4

5     5     5     5     5

f =

0     0.2500       1.0000        2.2500       4.0000

0     2.0000       8.0000      18.0000     32.0000

0    6.7500     27.0000       60.7500    108.0000

0   16.0000    64.0000    144.0000    256.0000

0   31.2500  125.0000    281.2500     500.0000

Simpson coefficients

The grid points for N = 5 and how these points relate to the Matlab matrices.

Calculation of [2D] Simpson coefficients

sc = np.ones(N)

R = np.arange(1,N,2);   sc[R] = 4;

R = np.arange(2,N-1,2); sc[R] = 2

scx, scy = np.meshgrid(sc,sc)

sc2D = scx*scy

Compute the integral

hx = (bx-ax)/(N-1); hy = (by-ay)/(N-1)

h = hx * hy / 9

integral = h*sum(sum(sc2D*f))

With only 5 partitions and 25 (5x5) grid points, the numerical estimate is 416.0, the same as the exact value.

Example 3   Double Integrals and Volumes     op001.py

To gain an intuitive feel for double integrals, the volume of the region enclosed by the area A is equal to the value of the double integral.

Volume V of a rectangular box

f(x,y) = k          height of box   k > 0

Base of box – the lower bounds (ax and ay) and upper bounds (bx and by) determine the area of the rectangular base of the box

Box    k = 1  ax = 0  bx = 1  ay = 0  by = 1  N = 299

Exact volume (analytical)   V = 1.0000

Simpson’s [2D] rule           V = 1.0000

Volume V of half box

f(x,y) = 1 - x

Base of box – the lower bounds (ax and ay) and upper bounds (bx and by) determine the area of the rectangular base of the box

ax = 0  bx = 1  ay = 0  by = 1

N = 299

Exact volume (analytical)  V = 0.50000

Simpson’s [2D] rule           V = 0.50000

Volume V of a part-bowl

f(x,y) = x2 + y2

Base of box – the lower bounds (ax and ay) and upper bounds (bx and by) determine the area of the rectangular base of the surface

ax = -1  bx = 1  ay = 0    by = 1

N = 299

Exact volume (analytical)  V = 1.3333

Simpson’s [2D] rule           V = 1.3333

Volume V over a rectangular base

Base of box – the lower bounds (ax and ay) and upper bounds (bx and by) determine the area of the rectangular base of the surface

ax = 0  bx = /2  ay = 0    by = /2  N = 299

Exact volume (analytical)

V = 1.000

Simpson’s [2D] rule

V = 1.000000000008578

Volume of a hemisphere using Cartesian coordinates

Volume of a hemisphere of radius a

Function

ax = -1  bx = 1  ay = -1    by = 1    a = 1

| Python plot | Matlab plot is better than Python plot. |
| --- | --- |

Exact volume (analytical)

V = 2.094395102393195

Simpson’s [2D] rule

N = 99       V =  2.094417986583109

N = 999     V =  2.094395646847362

We must have

The code to define the function and mask matrix:

f = np.real(a**2 - xx**2 - yy**2)

# Mask matrix M

M = np.ones([N,N])

M[xx**2+yy**2>1]=0

Volume V over a triangular base

ax = 0  bx = 1  ay = 0    by = 1    height h = 6

Exact volume (analytical)

V = 3.0000

Simpson’s [2D] rule

N = 299       V = 3.006673873549240

N = 999       V = 3.001944436635462

N = 999       V = 3.001944436635462

N = 2999     V = 3.000632027607761

Even with N = 2999 the calculation took less than 1.0 s on a fast Windows computer.

The differences between the exact and computed values is due to the rectangular grid and the condition on the function being zero when y > 1 – x

(y = 1 – x is a diagonal line and the grid is rectangular).

A logical Python function is used to define the function when

y > 1 – x. The code to define the function is

f = 6*xx**0*yy**0

f[yy > 1 - xx] = 0

Double Integrals In Polar Coordinates

https://tutorial.math.lamar.edu/classes/calciii/dipolarcoords.aspx

Double integrals in polar coordinates can be expressed as

where the cartesian and polar coordinates are

Example 4     area of circle of radius a         op002.py

p1 = 0      p2 = 2    q1 = 0     q2 = 2    a = 1    N = 999

Exact area           A =  = 3.141592653589793

Simpson’s [2D] rule  A = 3.141592653589793

Volume of a hemisphere of radius a     op002.py

ax = 0  bx = 2  ay = 0  by = 2  N = 299

Exact volume (analytical)        V = 16.755160819145562

Simpson’s [2D] rule                V =  16.755160819145555

Python code  op002.py

# Libraries

import numpy as np

from numpy import pi, sin, cos, linspace

from numpy.linalg import eig

from scipy.integrate import odeint

import matplotlib.pyplot as plt

import time

from mpl_toolkits.mplot3d import axes3d

tStart = time.time()

# >>>>> Input number of grid points: N must be odd

N = 999

# p rho /  q phi

# >>>>> Setup RP meshgrid

p1 = 0;  p2 = 2; q1 = 0; q2 = 2*pi

p = linspace(p1,p2,N)

q = linspace(q1,q2,N)

pp, qq = np.meshgrid(p,q)

# Mask matrix M

M = np.ones([N,N])

# Function

f = pp

# Simpson [2D] coefficients

S = np.ones(N)

R = np.arange(1,N,2);   S[R] = 4;

R = np.arange(2,N-1,2); S[R] = 2

scx, scy = np.meshgrid(S,S)

S = scx*scy

# Calculate integral

hp = (p2-p1)/(N-1); hq = (q2-q1)/(N-1)

h = hp * hq / 9

integral = h*sum(sum(pp*f*S))

print('\n Integral  =  ',integral)

xx = pp*cos(qq); yy = pp*sin(qq)

#%% GRAPHICS

plt.rcParams['font.size'] = 10

fig = plt.figure(figsize=(4,3))

ax = plt.axes(projection='3d')

ax.plot_surface(xx, yy, f, cmap='jet',

edgecolor='none', alpha=1,antialiased=True)

ax.set_xlabel('x', fontsize=12)

ax.set_ylabel('y', fontsize=12)

ax.set_zlabel('f', fontsize=12)

fig.tight_layout()

ax.view_init(17,-110,0)

# fig.savefig('a1.png')

#%%

tExe = time.time() - tStart

print('  ')

print('Execution time')

print(tExe)