DOING PHYSICS WITH PYTHON

COMPUTATIONAL OPTICS

RAYLEIGH-SOMMERFELD 1

DIFFRACTION INTEGRAL

BEAM PROPAGATION FROM AN APERTURE: MATHEMATICAL CONCEPTS

Ian Cooper

Please email me any corrections, comments, suggestions or additions:   matlabvisualphysics@gmail.com

DOWNLOAD DIRECTORIES FOR PYTHON CODE

Google drive

GitHub

emRS01.py

RAYLEIGH-SOMMERFELD DIFFRACTION INTEGRAL OF THE FIRST KIND

The Rayleigh-Sommerfeld region includes the entire space to the right of the aperture. It is assumed that the Rayleigh-Sommerfeld diffraction integral of the first kind is valid throughout this space, right down to the aperture. There are no limitations on the maximum size of either the aperture or observation region, relative to the observation distance, because no approximations have been made.

The Rayleigh-Sommerfeld diffraction integral of the first kind (RS1) can be expressed as

(1)

where  is the electric field at the observation point ,  is the electric field within the aperture and rPQ is the distance from an aperture point Q to the observation point P. The double integral is over the area of the aperture SA. The wavelength of the light and the propagation constant are

The irradiance I is proportional to the square of the magnitude of the electric field, hence the irradiance in the space beyond the aperture can be calculated by

(2)	        I0 is a normalizing constant

Fig. 1.  Geometry of the aperture and observation spaces.

The electric field  at the point P can be computed by evaluating the double integral (equation 1) over the aperture space Q  numerically using a two-dimensional form of Simpson’s 1/3 rule as given by equation 3 when arbitrary units are used for the electric field and irradiance

(3)

where Smn are the Simpson’s two-dimensional coefficients. Each term in equation (2) can be expressed by a square matrix and the matrices can be manipulated very easily in Python to give the estimate of the integral as indicated in figure 2.

Fig. 2.  Matrices used in computed the diffraction integral.

EXAMPLE    emRS01.py

Diffraction from a rectangular aperture

emRS01.py

# INPUT PARAMETERS

num = 30             # number for observation space

nP = num*4+1         # observation points for P:  format integer * 4 + 1

nQ = 159

# aperture points for Q  must be ODD

wL = 632.8e-9        # wavelength [m]

# Aperautre space: full-width [m]

aQx = 2e-4; aQy = 4e-4

#Observation spacehalf: half-width % zP   [m]

xPmax = 10*1500*wL; yPmax = 10*1500*wL; zP = 1 #55000*wL

# SETUP

k = 2*pi/wL            # propagation constant

ik = k*1j              # jk

# Initialise matrices

unit = np.ones([nQ,nQ])    # unit matrix

rPQ = np.zeros([nQ,nQ]); rPQ3 = np.zeros([nQ,nQ])

MP1 = np.zeros([nQ,nQ]); MP2 = np.zeros([nQ,nQ]); kk = np.zeros([nQ,nQ])

MP = np.zeros([nQ,nQ])

EQ = np.ones([nQ,nQ])

# Aperture space

xQmin = -aQx/2;  xQmax = aQx/2

yQmin = -aQy/2;  yQmax = aQy/2

xQ1 = linspace(xQmin,xQmax,nQ)

yQ1 = linspace(yQmin,yQmax,nQ)

xQ, yQ = np.meshgrid(xQ1,yQ1)

RQ = (xQ**2 + yQ**2)**0.5

#EQ[RQ > xQmax] = 0

# Observation space

xPmin = -xPmax; yPmin =  -yPmax

EP = np.zeros([nP,nP])+np.zeros([nP,nP])*1j

xP1 = linspace(xPmin,xPmax,nP)

yP1 = linspace(yPmin,yPmax,nP)

xP, yP = np.meshgrid(xP1,yP1)

# Simpson [2D] coefficients

S = np.ones(nQ)

R = np.arange(1,nQ,2);   S[R] = 4;

R = np.arange(2,nQ-1,2); S[R] = 2

scx, scy = np.meshgrid(S,S)

S = scx*scy

# % COMPUTATION OF DIFFRACTION INTEGRAL FOR ELECTRIC FIELD % IRRADIANCE-

for c1 in range(nP):

for c2 in range(nP):

rPQ = np.sqrt((xP[c1,c2] - xQ)**2 + (yP[c1,c2] - yQ)**2 + zP**2)

rPQ3 = rPQ*rPQ*rPQ

kk = ik * rPQ

MP1 = exp(kk)

MP1 = MP1 / rPQ3

MP2 = zP * (ik * rPQ - unit)

MP = MP1 * MP2

EP[c1,c2] = sum(sum(EQ*MP*S))

Irr = np.real(EP*np.conj(EP))

Irr = Irr/amax(amax(Irr))

indexXY = num*2+1

IY = Irr[:,indexXY]

IX = Irr[indexXY,:]

# Position of first zero in irradiance

x0 = wL*zP/(1*aQx)*1e3    # [mm]

y0 = wL*zP/(1*aQy)*1e3    # [m]