DOING PHYSICS WITH PYTHON

CONDUCTION OF THERMAL ENERGY

MODELLING ENERGY LOSSES FROM A ROOM

Ian Cooper

matlabvisualphysics@gmail.com

DOWNLOAD DIRECTORIES FOR PYTHON CODE

Google drive

GitHub

tp001.py

Thermal conduction: energy lost from a room through a brick wall slab and a glass window.

tp002.py

Thermal conduction: energy lost from a room through a brick wall with a wood insulation fibre.

tp003.py

Thermal conduction: energy lost from a room through a double brick wall slab with a wood fibre insulation filling the cavity between the bricks.

INTRODUCTION

The transfer of heat is very important in industry, in the home and in biological systems. At times it is desirable to obtain the maximum heat transfer, at other times to reduce it. It is essential that the thermal properties of materials from the lowest to the highest temperatures are known. Then systems can be designed to meet the requirements of maximum or minimum transfer of energy. For example, what is the purpose of the large cooling towers in a power station? How can we keep our home warm in winter? How does our body try to keep us cool during a hot summer day? On a very hot day you pick up two bars lying on the ground in the sun, one is made of wood, the other metal, what would you feel and why? In answering these questions, you have to consider, what produces a transfer of energy, and what causes a change

in temperature. Your answers may include the terms hot, cold, temperature, heat, energy, energy transfer, power, energy flux, and energy flux density but what do these terms mean? Can you clearly differentiate the concepts associated with each of these terms?

You can gain a deeper and better understanding of these concepts, by modelling the transfer of energy though a slab (material) by running the simulations with a wide range of input parameters.

Heat is a misleading term

Heat is the term used to describe the transfer of energy due to a temperature difference.

A better alternative is to simply use the term energy and so on most occasions the word energy will be used and not the term heat. The rate of energy transfer across a surface is be referred to as the power or energy flux. The energy flux density J is a very useful term because it is independent of the cross-sectional area A of the slab. The energy flux density J is defined as

(1)

We will model the one-dimensional heat transfer through a slab. The model can be used to compute both the temperature and the transfer of energy as functions of time and position.

The rate of energy transfer through a slab is dependent upon the thermal conductivity k of the material, the cross-sectional area A of the slab and the temperature gradient as described by the differential equation

(2)

Using equations (1) and (2), the energy flux density J is

(3)

When energy is transferred to or from an object its temperature may change. The change in temperature dT depends upon the object’s mass m, its specific heat capacity c and the net amount of energy Qnet gained or lost by the object. The change in temperature dT is given by

(4)

and the time rate of change of temperature is

(5)

We can consider the width of the slab to be made up of many elements of length dx. The density of an element is  and its volume is . Then the mass m of an element is

(6)

Combing equations (1), (4) and (5), the time rate of change of the temperature of an element is

(7)

where the net energy flux density  is given by

(8)

Jcond is the contribution to the energy flux density due to the conduction of energy through the slab, Jint is the internal heating of the slab (for example, an electric heating element) and Jenv is the due to the transfer of energy to and from an element to the surrounding environment (for example, by thermal radiation or convection).

We can find the temperature T and energy flux density Jnet as functions of time t and position x through the slab by solving the two differential equations (3) and (7) using a finite difference approach

(3)

(7)

For convenience, to simply the notation, we will use J to represent the energy flux density due to conduction .

Consider a slab of width L divided into N elements and the width of each element is .  It is assumed all the properties of an element are uniform throughout the volume of the element. For the nth element (n = 1 , 2, …, N), it is characterised by its thermal conductivity k(n), specific heat capacity c(n) and its density (n). The time dependent variables are its temperature T(t, n) and energy flux densities Jint(t, n) and Jenv(t, n). All these variables are measured at the mid-point of the element  .

Energy is transferred across the interface between elements by the process of conduction. The energy flux density into the nth element is J(t, n) at position  and the energy flux density existing the element is J(t, n+1) at position .  Figure 1 shows the parameters for the element with index n at time t.

Fig. 1.   Parameters for the element with index n at time t.

The differential equation (3) for the energy flux density can be expressed as a finite difference equation for the nth element at time t as

(8)

The differential equation (7) for the temperature can be expressed as a finite difference equation for the nth element at time  as

(9)

We can solve this pair of finite difference equations (8) and (9) given the initial values at time t = 0 and the boundary conditions imposed on the ends of the slab.

For stability of the numerical method, we have the condition

The smaller the value dx for the increment in x then the smaller the time step dt in the simulation.

For composite materials made of more than one material, it may not be possible to use actual values for the heat capacity and density as they the set time scale in solving the differential equations. The steady-state temperature profile along a slab and the energy flux density only depend upon the width and thermal conductivity of the materials making up the composite material, therefore, values of the heat capacity and density can be chosen to give an appropriate time step.

SIMULATIONS

Consider a room of a house where the outside temperature is  5 oC and the inside room temperature is 25 oC. We can model the loss of energy from the room to the outside through a brick wall and through a glass window by conduction. The cross-section area of the slab for the brick and glass is 1.0 m2. The initial temperature for each element of the slab is 5 oC except the end element which is set to the room temperature, 25 oC.

Fig. 1.  Energy flux density J is transmitted from room to the outside of the house because of the temperature difference due to the boundary conditions TA = 5 oC (first element of slab) and TB =25 oC (end element of slab).

Simulation 1      Energy flux density through brick and glass tp001.py

The inputs are:

#%% INPUTS

# Input flagS: building brick 1 or glass window 2

flagS = 1    # <<<<<

# Building brick

if flagS == 1:

nx = 121      # number of spatial elements (101)

nt = 30000    # number of time steps (50000)

L = 0.08      # width

k1 = 1.33     #  thermal conductivity W/(m.K)

rho1 = 1600   #  density kg/m**3

c1 = 900      #  specific heat capacity J/(kg.K)

# Glass window

if flagS == 2:

nx = 121      # number of spatial elements (101)

nt = 30000    # number of time steps (50000)

L = 3e-3      # width

k1 = 0.8      #  thermal conductivity W/(m.K)

rho1 = 2600    #  density kg/m**3

c1 = 840     #  specific heat capacity J/(kg.K)

Fig 1.1 Pcolor plot of the steady-state temperature profile across the slab.

Fig. 1.2.  Temperature distribution through the slab. The steady-state temperature profile is reached in about 1 seconds for the glass but about 1 hour for the brick.

Steady-state temperature gradients:

brick = -2.5x102 oC.m-1

glass  = -6.7x103 oC.m-1

Glass temperature gradient is ~27x greater than the brick

Fig. 1.3.  Energy flux density through the brick and glass slabs. A constant energy flux density through the glass takes about 10 seconds and about 1 hour for the brick.

Steady-state energy flux density J

Jbrick = -0.34 kW.m-2      Jglass = -5.4 kW.m-2

The rate of energy lost from the room to the outside is 18x more through the glass than the brick.

Fig. 1.4.  The time evolution of the temperature and energy flux density at the centre of the slab. The steady-state is reached for the glass much quicker than for the brick.

Energy is transferred through the slabs from the hot reservoir to the cold reservoir. Initially a large amount of energy is transferred through the slab in the direction of decreasing temperature, heating the slab which results in the rise in temperature along its length. Finally, a steady-state situation is established with a linear variation in temperature through the slab and a constant energy flux density through it.

The significance of the minus sign ( J < 0 ) is that the energy transfer is in the direction of decreasing temperature, i.e., heat always flows spontaneously from the location of higher temperature to locations of lower temperature.

For an energy efficient home, the lower the rate of energy lost from the interior to the external the better. We can model a double brick wall (L  8.0 mm  16 mm) and a better glass window for insulation L  3 mm  10 mm) to achieve better insulation to reduce heat losses.

Brick: L = 0.08 m    Jbrick = -0.34 kW.m-2

L = 0.16 m    Jbrick = -0.17 kW.m-2

The double brick slab halves the energy lost.

Glass: L = 3.0 mm  Jglass = -5.44 kW.m-2

L = 10 m       Jglass = -1.63 kW.m-2

The thick glass slab reduces the energy lost by a factor of 3 but the energy loss is still much much greater than the loss of energy through the brick.

Simulation 2    Conduction through a composite slab

Brick and Wood Fibre Insulation

tp002.py

Energy flow calculations often involve composite walls that are composed of more than one material. For insulation purposes, these composite walls have a layer of a material with low thermal conductivity.

If the slab is only brick of thickness 75 mm thick, then the temperature difference of 20 oC gives a steady-state energy flux density of 360 W.m-2.  So, considerable energy losses occur through the brick walls of the room.

To improve the insulation of a house, an insulating fibre can be inserted into wall cavities. We can model this by considering a composite slab of brick with thickness 75 mm and a wood fibre insulating material of thickness 75 mm. The parameters for wood fibre are:

thermal conductivity k = 0.038 W.m-1.K-1

density =200 kg.m-3

specific heat capacity c = 2100 J.kg-1.K-1

The slab’s temperature will evolve with time from its initial temperature to a steady-state situation when the energy flux density is uniform through the width of the slab (the rate of energy transfer through the two materials are equal) where

(2.1)

From equation 2.1 it is easy to show that the theoretical temperature at the junction between the brick and fibre is

(2.2)

The theoretical value agrees with the numerical prediction. The larger drop in temperature occurs across the fibre because the fibre has a much lower the conductivity than the brick.

Fig. 2.1.  Temperature across brick-fibre slab.

Fig. 2.2.  Steady-state temperature distribution through the brick-fibre slab.

Fig. 2.3.  The temperature evolution at the centre of the brick and centre of the fibre.  Steady-state values are obtained only after many hours, especially for the brick.

Fig. 2. 4. Time evolution of the energy flux density through the brick and the fibre. It takes much more time for th steady-state energy flux density through the brick to be reached compared with the fibre.

The loss of energy through the brick is 360 W.m-2 where after a layer of wood fibre insulation is added, there is a significant reduction in the energy lost through the insulated wall. The energy flux density drops to 10 W.m-2.

Simulation 3        Filled cavity double brick wall

We will consider a specific example of a composite wall of double brick with the cavity between the brick walls filled with a wooden insulation fibre. The width of the filled cavity is 20 mm and the width of the slab is 170 mm.

Fig 3.1.   The temperature distribution through the wall segment. The large temperature gradient occurs across the wood fibre filled cavity.

Fig. 3.2.  Time evolution of the energy flux density. The steady-state energy flux density is -32 W.m-2.

The energy lost through a double brick wall (150 mm width) is 180 W.m-2. By adding the wood fibre insulation (20 mm width) into the cavity reduces the energy loss to 32 W.m-2.